# Operating Safety Manual: Agipix - Autonomous Aerial Development platform

## System specifications

| Characteristic | Value |
| --- | --- |
| Weight | 1254 g |
| Peak Thrust | 37.6 N |
| Thrust to Weight | 2.98:1 |
| Max Payload | 1252 g |
| Dimensions | 348 x 306 x 142 mm |
| Low level controller | PX4 |
| High level computer | Nvidia Jetson Orin NX (Connect tech Hadron career) |
| CPU mark | 3170 on glmark2-es2 (100 TOPS (INT8)) |
| GPU | Yes |

## Isometric view

## Exploded view

Safety switch [1]

Jetson Orin NX

Landing gear

Lipo battery

ESC

RealSense D455

Propeller guards

Carbon fibre top plate

PX4

RealSense T265

Brushless motor

Arm/kill switch [2]

Live telemetry

Emergency Land [4]

Force Hover [5]

Offboard enable switch [3]

## Safety layers

### [HW] Hardware safety switch

This is the topmost layer and overrides all below. Motors can not be powered unless this is activates. A safety switch [1] is a control on the vehicle that must be engaged before the vehicle can be armed, and which may also prevent prearming (depending on the configuration).

### [SW] Prearm checks.

Low level controller runs pre-arm checks for sensor health, localisation accuracy, battery level etc. Vehicle can not be armed if the check fails.

### [HW] Remote Arm/Kill switch.

An arming switch [2] is a switch or button on an RC controller that can be used to arm the vehicle and start motors (provided arming is not prevented by a safety switch).

### [HW] Emergency Land/ Force hover switch

These switches in the RC controller ([4] and [5]) overrides offboard control at operators will.

### [HW] Remote Offboard enable switch.

This switch on the RC controller [3] has to be enabled to go to the software-controlled mode. All above must be satisfied for activation.

### [SW] System health monitor

A health monitor is implemented in the high-level controller that monitors live telemetry and disables offboard control and goes back to force hover if safety parameters fall below thresholds.

### [SW] ROS2 Heartbeat message

The above health monitor broadcasts a heartbeat at 5 Hz to the ROS 2 system. This informs the connectivity and health of the vehicle to the system wide safety measures.

### [SW] Teleoperator node Emergency Land/ Force hover option

The ROS2 teleoperator node also has Emergency Land/ Force hover options that can disable offboard control during teleoperation

## General Operation

- Operator connects the battery to power the system.

- Operator checks High level controller connection to the system after boot.

- Operator initiates Visual odometry to the low-level controller.

- Operator enables Safety switch [1] when prearm checks enable position control (Can be monitored in live odometry)

- Operator initiates ROS2 teleoperation.

- Operator toggles arm switch [2]

- Operator toggles offboard switch [3]

- Teleoperator now has control of the system.